|  | Engine 746cc Tohatsu 2 strokes, water-cooled gasoline engine Authorized output 55PS (40.5kW) Pump performance 2050 liter / min. at 0.6MPa 1800 liter / min. at 0.8MPa 1500 liter / min. at 1.0MPa Dry weight 94kg |
| --- | --- |

- Features of VC82ASE

More attractive features and functions are added to maximize efficiency of fire services.

1. Light weight and compact 
Engine and pump castings are made of anti-corrosive aluminum-alloy to reduce weight and prolong 
working life. Compact unit in its class fits existing locker space, also can be carried by 2 firemen.

2. High pump performance 
Both engine and pump are designed and developed specifically for fire-pump application by Tohatsu, 
ensuring higher pump performance for its class.

3. Reliable engine with optimal engine start-up 
The engine of VC82ASE is originally designed by Tohatsu for this model. Equipped with oil pump, 
gasoline and engine oil are mixed automatically at a precise ratio to engine speed without mixing fuel 
and oil by operator.

4. Priming 
Quick priming is provided by operating Rotary Vane Vacuum Pump. 
　　　Prime 1meter in 4seconds (6m suction hose) 
　　　Prime 3meter in 5.5seconds (6m suction hose) 
　　　Prime 6meter in 8.0seconds (6m suction hose).

5. Centralized control 
All gauges, throttle dial, control switches, and other warning lamps are grouped on the control panel 
for easy operation.

6. Cooling water recirculation system 
Engine cooling water is returned to the pump eliminating discharge outside of the pump, and making 
surrounding area soaked.

7. Unique auto choke carburetor 
This device helps engine start easier in all seasonal temperature conditions.

8. Engine overheat protection sensor 
Overheat protection is provided on this model to safeguard the engine from cooling water loss, and to 
protect engine when water is not discharging. This device stops the engine automatically when engine 
temperature reaches 80℃. Reset switch is provided to release the sensor after it stops the engine.

9. Large capacity fuel tank 
Integrated fuel tank with large capacity of 18liters ensures the fuel supply will last for 1 hour at maximum 
throttle opening and load.

10. 90 degree rotating delivery outlets

11. Electric starter with back up recoil starter 
An electric starter and back-up recoil starter are installed to guarantee starting engine under any 
conditions.

12. Maintenance free battery 
Equipped with sealed type and maintenance free battery, once the battery is filled with a specified electrolyte, maintenance of electrolyte is not required.

Accessories

1. Battery charger

2. Floodlight projector

3. Plastic toolbox

Adapters:

Some different type of adapters for suction port and discharge ports are available.

| Specifications |  |  |  |
| --- | --- | --- | --- |
| Model: |  | VC82ASE |  |
| Engine: | Type: | Tohatsu 2-stroke, 2-cylinder, water cooled gasoline engine |  |
|  | Bore & Stroke: | 78mm x 78mm |  |
|  | Piston displacement: | 746cc |  |
|  | Authorized output: | 55PS/ 40.5 kW |  |
|  | Fuel tank: | 18 liters |  |
|  | Fuel consumption: | 20liters/h |  |
|  | Ignition: | Flywheel magneto (C.D.Ignition) |  |
|  | Starting system: | Starter motor and automatic recoil system |  |
|  | Lubrication: | Auto mixing (Separate oil tank) |  |
|  | Choke system: | Auto choke |  |
|  | Spark plug: | NGK BP7HS-10 |  |
|  | Light bulb: | 12V-35W |  |
|  | Battery: | 12V-16Ah/5h |  |
| Pump: | Type: | Single suction, single stage, high pressure turbine pump |  |
|  | Delivery outlet: | JIS fire thread (B-9912), 2-1/2" (65mm) male |  |
|  |  | BSP 65mm (G 2-1/2") male |  |
|  | Suction inlet: | JIS fire thread (B-9912), 3-1/2" (90mm) male |  |
|  |  | BSP 100mm (G 4") male |  |
|  | Priming: | Rotary-vane vacuum pump (oilless type) |  |
| Pump performance: (Suction height: 3m) |  | 2,050 liter/min. at 0.6 MPa |  |
|  |  | 1,800 liter/min. at 0.8 MPa |  |
|  |  | 1,500 liter/min. at 1.0 MPa |  |
| Dimensions: |  | Length: 742mm x Width: 682mm x Height:760mm |  |
| Mass: | Dry: | 94kg |  |
|  | Wet: | 110kg |  |
| Max. suction height: |  | Approx. 9m (29.5 feet) |  |

Back top